Department of Mechanical and Aerospace Engineering

ME-UY 3231 - Structures Practicum

Fall 2025

Pin Group Activity

Date of Lab Report Submission:  12/05/2025

Group 2

| Shahzaib Zamir | sz3947 |
| --- | --- |
| Sahil Krishnani | acp8668 |
| Kevin Rodriguez | adh10036 |
| Jayden Senior | amt9940 |

Table of Contents

1. Introduction	3

2. Pin Group Analysis - Code	3

3. Fixture Design Overview	3

4. FEA Numerical Analysis	4

5. Strain Comparison (Predicted vs. Measured)	4

6. Discussion	5

7. Appendix	5

# 1. Introduction

The purpose of this activity was to design and analyze a 3D-printed pin-shear test fixture capable of evaluating the shear strength of PTFE pins in double shear. The fixture needed to meet strict geometric constraints: an overall clevis-to-clevis spacing of 8.85 inches, a maximum thickness of 1 inch, and a total printed mass below 0.5 Ib. In addition, the two pin holes were required to lie on the load axis so that the applied tensile force produced pure shear in both 3/16 inch PTFE pins.

A two-piece interlocking fixture was then modeled and assembled in CAD, with pin holes aligned to create double shear. Finite Element Analysis (FEA) was performed to predict strain distribution in the gage section and the stresses in both the print and the pins under a 222-Ibf tensile load. Finally, the experimental strain was calculated using measured strain-gage resistance values and compared to the FEA prediction.

# 2. Pin Group Analysis - Code

To evaluate the strength of possible pin configurations, a Python script was developed to compute the total double-shear capacity for groups of 3-8 pins selected from a predefined grid of 15 candidate pin sites, each with a fixed diameter. The script computes the predicted failure load Pfail for every pin combination. It defines 15 potential pin sites, each with fixed coordinates (x,y) in inches:

- Row 1 (y=0.0): sites 1-3 at x=1.5, 2.0, 2.5

- Row 2 (y=0.5): sites 4-6

- Row 3 (y=1.0): sites 7-9

- Row 4 (y=1.5): sites 10-12

- Row 5 (y=2.0): sites 13-15

The script accounts for a vertical load applied at a clevis point (xL, yL) = (0, 3.25), creating an eccentric load relative to the pin-group centroid. It:

- Computes the centroid of the chosen pin group and shifts coordinates so the centroid is at the origin.

- Computes ρI2 where ρi is the radial distance of each pin from the centroid.

- For a vertical load applied at (0, 3.25), computes the induced moment per unit load, MP=-(xL-xc).

- Distributes the combined direct-shear and tangential shear to each pin, forming an effective per-pin shear demand factor Si.

- For each combination, calculates:

- Double Shear area: A=πd22

- Double-shear capacity: Ci=τallowA

- Limiting failure load for each pin: Pfail, i=CiSi

- Group failure load: Pfail, group=min Pfail,i

The analysis uses an allowable shear stress of 6,000 psi for PTFE pins. The code automatically identifies the best, worst, and median combinations based on largest to smallest Pfail. These results helped confirm that the chosen two-pin fixture for experimental testing would experience forces below the material limits of the printed ABS, avoiding premature fixture failure.

A table summarizing the best, worst, and middle groups is provided in the Appendix along with the full Python code.

Extra Credit Section: How AI Was Used to Solve the Problem? After determining the necessary calculations and methodology for doing the three pins, the results along with the pin locations and diameters was provided to ChatGPT model to recreate the load calculations for the three-pin combos and asked to replicate where it checks all three-pin combinations and make sure it outputs similar results to what was provided as the best and worst three-pin combination. After some trial and error working back and forth to make sure that the code outputted the correct combinations, the code was expanded by AI for combinations from 4 to 8 and to tabulate all the results into a CSV to be outputted accordingly.

# 3. Fixture Design Overview

A two-piece fixture was designed so that the upper and lower plates could interlock through a matching rectangular male-and-female interface. This configuration ensured repeatable alignment of the two pin holes, allowing the 3/16-in pins to act in double shear during testing. The geometry was inspired by the standard tensile “dogbone” style, but modified to satisfy the requirements:

- Height between clamping holes: 8.80 in

- Slot width: 1.18 in, matching the male width of the mating part.

- Thickness: 0.5 in

- Upper loading hole: 0.30 in

- Shear pins: two 0.188-in pins

- Interlocking depth: 1.18 in

Smooth fillets were incorporated at the transitions to avoid stress concentration.

# 4. FEA Numerical Analysis

Finite Element Analysis was performed in ANSYS to predict the stress and strain response of the fixture under the same loading used in the physical experiment. The goal was to ensure that the printed ABS components remained elastic and that the pins experienced realistic double-shear loading.

- Material: ABS

- Pins: Solid cylinders representing dowels

- Contacts: Bonded pin-hole interfaces to transmit shear without slip

- Mesh: Tetrahedral elements with automatic global element size

The following are the boundary conditions:

- The top clevis hole was fully fixed.

- A 222-Ibf downward force was applied to inner surface of the bottom clevis hole.

The following quantities were extracted:

- Equivalent elastic strain in the gauge region

- Von Mises stress in the fixture

- Shear-dominant stress in the pins

The results were later compared directly with experimental strain measurements to evaluate accuracy.

# 5. Strain Comparison (Predicted vs. Measured)

To evaluate the structural response of the printed fixture, strain was obtained from two sources:

- Numerical prediction (FEA)

- Experimental strain gage measurement

A tensile load of 222 Ibf was applied in both cases.

From the ANSYS Equivalent Elastic Strain contour: the maximum strain in the gauge region is ≈1.35⋆10-3inin. Thus the numerical prediction for the tapered gauge section is ≈0.00138 inin.

The experimentally measured strain using the following strain gage resistance values are:

- R0=120.8Ω

- Rf=121.7Ω

- Gage factor GF=2.08

Strain=(Rf-R0)R0⋅GF=121.7-120.8120.8⋅2.08=0.00358inin

The measured strain is significantly greater than the numerical prediction. This discrepancy is expected due to several factors:

- Material stiffness varies with print direction, while FEA assume isotropic properties. FEA also assumes 100% infill.

- Printed surface textures affect adhesion and strain transfer.

- Load eccentricity increases bending strain in the gauge region.

Despite quantitative differences, both results indicate that the fixture remained elastic under the 222 Ibf load, and strain is concentrated near the pin holes and the tapered transition, consistent with the expected stress flow. A full set of stress and strain plots is provided in the Appendix.

# 6. Pin-Group Optimization Results

Using the provided Python script:

# 6. Discussion

The pin-loaded fixture successfully transfer load through the two 3D-printed plates, and the inserted pins, allowing both strain gage data and FEA predictions to be compared. The stress contours showed that the highest stresses formed around the lower pin interface, consistent with classic double-shear behavior, while the tapered gauge region carried uniform axial strain. Although the experimental strain (0.00358 in/in) was higher than the predicted strain (0.00138 in/in), the relationship between load and deformation followed the expected trend.

From a design standpoint, the custom dogbone-shaped plate provided a controlled region of deformation and ensured that the load path remained centered through the two pins. The FE model confirmed that the plate thickness and geometry were sufficient to avoid yielding, and the measured stress levels in the pins were well below any failure thresholds.

# 7. Individual Responsibilities

| Name | Responsibilities |
| --- | --- |
| Shahzaib Zamir | Report, Code, Calculations |
| Alex Trujillo | Test fixture, Report, Code |
| Andy Park | Test Fixture, 3D Printing, CAD |
| Anthony Huanca | Test Fixture, Report, CAD |

# 8. Appendix